# Battery Cell Specification (Summary)

This document summarizes the specification items of the master standard document in narrative form.

## 1. Electrical Specifications

This specification conforms to Battery Approval Specification ver 4.7 (SEC Req. ver.4.7).

The rated charging voltage is 4.55V.

The discharge cut-off voltage shall be 3.0V.

The nominal voltage is 3.85V.

The maximum charging current shall not exceed 1.495A.

The evaluation ambient humidity shall be within the range of 33 to 53 %RH, with a center value of 43 %RH.

Charge temperature ranges:
-5~5℃, 0.1C(4.55V)
5~12℃, 0.3C(4.55V)
12~15℃, 0.7C(4.55V)
15~45℃, 1.2C(4.20V)

## 2. Key Specification Table

| Item | Specification | Unit |
| --- | --- | --- |
| Nominal capacity | 1150 | mAh |
| Rated capacity | 1150 | mAh |
| Standard charging current | 230 | mA |
| Maximum discharging current | 1150 | mA |
| Standard ambient temperature | 21 ~ 29 (center 25) | °C |

* Items not specified in this document shall be agreed upon separately.